# MATERIAL  SAFETY  DATA  SHEET

SECTION  1.  PRODUCT IDENTIFICATION

Trade name					COP-R-BOTE

Chemical Name & Synonyms			Copper filled epoxy resin / amine hardener two

component kit

Manufacturer					BoatCraft Pacific Pty. Ltd.

22 Babdoyle St

Loganholme, Queensland 4129

Australia

Emergency Contact Telephone			07 3806 1944,  (ah) 07 3299 1468

UN No.   	1263				Hazchem   		3[Y]

DG Class	3.2				Poisons Schedule	5

CAS No.	Mixture				Pkge Grp		III

Intended usage					A two component permanent marine antifouling coating

Based on pure copper metal as active ingredient.

SECTION  2.  INGREDIENTS

Epoxy resin					CAS No. 25068-38-6		10 – 20 %

Metallic copper									60 – 70 %

Cycloaliphatic amine/epoxy resin adduct		CAS No.   N/A			0 –10 %

Amidoamine					CAS No. 112-57-2		0 –10 %

Xylene solvent					CAS No. 1330-20-7		0 – 10%

SECTION 3.  PHYSICAL & EXPOSURE DATA

Physical data

Appearance / odour				Viscous liquids, solvent odour

Boiling range					138 – 143 deg C

Flash Point					27 deg C

Specific gravity					2.8 (Part A),  1.0 (Part B)

Vapour pressure				6 mm at 20 deg C

Vapour density (air = 1)				3.66 at 20 deg C

Volatile component  (% vol)			0 – 10 %

pH						11 - 13

Solubility with water				Insoluble

Explosive limits					LEL 1.1% v/v, UEL 7.7% v/v

Product Exposure Limits

TLV TWA epoxy resin				No value assigned by NHMRC

TLV TWA cyclophatic amine			No value assigned by NHMRC

TLV TWA amidoamine	No value assigned by NHMRC – short term exposure considered to be practically non-harmful

TLV TWA xylene					100 ppm, 435 mg/cu m

Toxicity and Irritation Data

LD(50) for ingestion (rat) – epoxy resin			5 gm/kg

– amines			3.5 gm/kg

- xylene			5 gm/kg

SECTION 4.  HEALTH HAZARD

Symptoms of Exposure:  Acute and Chronic Effects

Ingestion:

Strong burning sensation, possible vomiting.  Can result in irritation of the gastrointestinal tract and mucous membranes.  Considered as an unlikely route of entry in commercial/industrial environments.  Xylene tends to break up into foam if the patient vomits and may be aspired into the lungs.

Eye contact:

The amine hardener component may be severely irritating to the eyes and may burn eye tissue.  High concentrations of vapour may cause irritation.

Skin contact:

Liquid may be irritating to the skin and may be capable of causing allergic skin reactions.  Repeated or prolonged contact may lead to dermatitic effects and may cause sensitisation in some individuals.

Inhalation:

The vapour is mildly irritating to the mucous membranes and respiratory tract.  Frequent or prolonged exposure can cause respiratory irritation, somnolence and narcosis, and may cause sensitisation in some individuals.  Overexposure symptoms include headache, swelling and congestion of the eyes and sinuses.

Systemic and other effects:

The ingredients may cause adverse effects including possible skin  sensitisation.  Symptoms include itching rash and respiratory congestion.  Xylene solvent in karge quantities may cause gastrointestinal irritation, CNS depression, and long term liver and kidney damage.

Emergency & First Aid Procedures

Ingestion:

DO NOT INDUCE VOMITING.  If conscious give water or milk to rinse out mouth and drink.  Provide liquid slowly but as much as casualty will drink.  Transport to hospital or doctor without delay.   Activated charcoal may be helpful.

Eye contact:

Immediately hold the eyes open and irrigate eyes with running water for at least 15 minutes.  Ensure irrigation under eyelids by occasionally lifting the upper and lower lids.  Transport to doctor or hospital without delay.  Removal of contact lenses after an eye injury should only be undertaken by skilled personnel.

Skin contact:

Immediately remove all contaminated clothing including footwear.  Wash affected areas thoroughly with detergent hand cleaner and rinse with plenty of water.  Seek medical attention in event of irritation.

Inhalation:

Remove to fresh air, lie patient down, keep warm and rested.  If breathing is shallow or has stopped, ensure clear airway and apply resuscitation.  Administer oxygen.  Transport to hospital or doctor.

Notes to physician:  Treat symptomatically.

SECTION 5.  PERSONAL PROTECTION & HANDLING

Protective Equipment

Eyes:			Goggles or face shield

Hands/feet:		Rubber gloves, full overalls, safety shoes

Respiratory:		Ensure adequate ventilation.  Wear organic vapour respirator or self

contained breathing apparatus in enclosed areas.

Remove and wash all contaminated clothing and equipment.

Handling Procedures

Handle in well ventilated area.  Always observe conditions of good industrial hygiene and safe working practice.

SECTION 6.  FIRE & EXPLOSION

Stability

Stable for minimum 2 years at room temperature.  Excess heating over long periods will degrade ingredients.  .

Flammability

Products will support combustion.  Explosive air-vapour mixture may form.  Keep away from heat, naked flames, or sparks.

Hazardous Decomposition Products

On burning will emit toxic fumes of oxides of nitrogen, carbon monoxide and carbon dioxide.

Hazardous Polymerisation

Will not occur in absence of contact with reactive elements or unless the separately packed components are mixed in large quantities.  This will cause irreversible polymerization with considerable heat build-up.

Incompatibility

Avoid contact with water, strong acids, alkalis, oxidising materials.

Fire Fighting

Toxic fumes will be evolved when this material is involved in a fire.  Fire fighters must wear full protective clothing and self contained breathing apparatus.

Extinguishing Media

Foam, carbon dioxide, dry chemical powder.

SECTION 7.  STORAGE & TRANSPORT

DG Class				3.2

Packaging Group & Label		III

Suitable Containers			Pre-packaged

Storage Procedures			Containers will develop pressure at high

temperatures.  Store at room temperature under

cover in accordance with AS1940 and State Poisons

Acts.

SECTION 8.  SPILLS  &  DISPOSAL

Minor spill		Absorb into waste cloth or other suitable absorbent.  Avoid

contact with skin or eyes.

Major spill		Contain with sand or earth, absorb with suitable absorbent, collect

and seal in properly labelled drums for disposal.  Prevent run-off

into drains or waterways.

Disposal		Suitable for incineration by approved agent or bury in approved

landfill according to local regulations.

SECTION 9.  AUTHORISATION

Name:			B.H. McConkey

Title			Technical Director

Issue Date		September 11, 2000